TRAFFIC INDUSTRIES

# Traffic barriers

SPECIFICATION TEXT (draft)

# 1. Subject and scope

This specification covers the supply of traffic barriers. The function and place of use follow the definition below:

The traffic barrier is used to separate a work area or a temporary route from traffic with a visual-physical line. The traffic barrier travels empty and is filled with water or sand on site; its coupling ends join the units into one line. It transports light, sets up fast and redeploys. Its top takes a warning lamp, showing the line at night too. The traffic barrier is used on road works, around site perimeters, at events, and at car park and facility entrances.

# 2. Features

The feature table is given in writing with the quotation.

# 3. Dimensions and classes

Body length: 1300–2000 mm (market classes)

Height: 650–1200 mm

Base width: 370–800 mm

Empty weight: 6–41 kg

Water ballast: 60–400 kg

Material: UV-stabilised PE / HDPE

Fill limit and operating weight: The maximum permitted fill volume and the filled operating weight are stated on the model datasheet; the fill mark on the body is not exceeded. In frost conditions the fill choice follows the datasheet instruction

Note: values are production-programme and market-standard ranges; order dimensions are fixed in the quotation.

# 4. Documentation

Manufactured to the order's specification in the Turkish market. In US language the water-filled traffic barrier is described by the longitudinal channelizing device definition of MUTCD Section 6K.10 (it may be hollow and water-ballasted). These devices are not protection barriers; they are not used to shield obstacles or protect pedestrians or workers. Where impact containment is required, EN 1317-classified systems are a separate conversation. Test results and certificates are shared with the report and the document.

Test results and certificates are shared only with the report and the document; no class name is cited without its document.

# 5. Packing and delivery

Ships empty — the container is limited by volume, not weight · Bodies nest or stack · Connecting pins travel in separate cartons · Freight cost is markedly lower than for concrete

# 6. Quotation and contact

Quotation requests go through trafficindustries.com; the quotation is prepared the same day against quantity, variant and delivery terms.